|  | STANDARDS ASSOCIATION OF ZIMBABWE |
| --- | --- |

DRAFT FOR 	PUBLIC COMMENT

LATEST DATE FOR RECEIPT OF COMMENTS: 2025-05-16

Our ref: ZWS 545 Draft Number: ZWS 545 Date: 2025-03-24

TECHNICAL COMMITTEE: AG 005 – ANIMAL FEEDING STUFFS

DRAFT ZIMBABWE TEST METHOD FOR ANIMAL FEEDING STUFFS – DETERMINATION CALCIUM CONTENT - TITRIMETRIC METHOD

This draft is now available for public comment. Your views and technical comments on it would be appreciated. If you have no specific comments to make but find it generally acceptable it would be helpful if you would notify us accordingly.  Suggestions entailing revisions of the text should indicate the preferred wording using the attached template.  The relevant clause number should be quoted against any comment.

All comments should be sent to the Committee Secretary Ms E Pindura at the address shown below.

Standards Association of Zimbabwe

P O Box 2259

Harare

E-mail: epindura@saz.org.zw

Website:  www.saz.org.zw

THIS IS A DRAFT AND MUST NOT BE REGARDED OR USED AS A ZIMBABWE STANDARD.

Copyright Users of Zimbabwe National Standards are reminded that copyright subsists in all ZWS publications. No part of this publication may be produced in any form or by any means, electronic or mechanical, including photocopying or posting on the internet or an intranet, without prior permission in writing from the Director General of the Standards Association of Zimbabwe. Orders for all national, international and foreign standards publications should be addressed to: Director – Standards Development, Information and Training Standards Association of Zimbabwe P O Box 2259 Harare Northridge Park Borrowdale Tel: 885511-2, 882017-9, 882021-2 Fax: 882020 E-mail: info@saz.org.zw Website: www.saz.org.zw

It is important that users of Zimbabwe Standards should ensure that they are in possession of the latest amendments or editions.

AMENDMENTS

| No. | MD No. | Date of Issue | Text affected |
| --- | --- | --- | --- |

Zimbabwe Standards are revised, when necessary, by issuing either amendments or revised editions.  Suggestions for improvements will be welcome at all times.

Contents																Page

Foreword		v

1.	Scope 		1

2.	Principle 		1

3.	Reagents		1

4.	Apparatus		2

5.	Sampling		2

6.	Procedure 		2

7.	 Expression of Results 		3

8.	Notes on Procedure		4

9.	Test Report 		4

Foreword

This Zimbabwe Standard Test Method ZWS 545: 2025: Animal feeding stuffs - Determination of calcium content - Titrimetric method, was revised by technical committee AG 005- Animal feeding stuffs, under the general direction of the Food and Agriculture (FA) Standards Sector.

First published 1996, as ZWS 545,

First Reprint 2015, as ZWS 545,

First Revision 2025, as ZWS 545.

This standard replaces ZWS 545:1996 which is now withdrawn.

This standard makes reference to the following publication:

ZWS 530	:	Animal feeding stuffs – Preparation of test samples and sampling.

ZIMBABWE STANDARD TEST METHOD

FOR

ANIMAL FEEDING STUFFS - DETERMINATION OF

CALCIUM CONTENT - TITRIMETRIC METHOD

(First Revision of ZWS 545:1996)

- Scope

This Zimbabwe Standard specifies a titrimetric method for the determination of the calcium content of animal feeding stuffs.

The method is applicable to all animal feeding stuffs having calcium contents greater than 1 g/kg.

Note. The title of the publication referred to in this standard is indicated in the Foreword.

- Principle

Ashing of a test portion, treatment of the ash with hydrochloric acid and precipitation of the calcium as calcium oxalate. Dissolution of the precipitate in sulfuric acid and titration with standard volumetric 
potassium permanganate solution of the oxalic acid formed.

- Reagents

All reagents shall be of recognized analytical quality. Distilled water or water of at least equivalent purity shall be used.

- Hydrochloric acid, approximately 30 % (by mass fraction)            (Q20 = 1,15 g/ml).

- Nitric acid, concentrated (Q20 = 1,40 g/ml).

- Sulfuric acid, approximately 20 % (by mass fraction                 (Q20 = 1,13 g/ml).

- Ammonia solution, approximately 33 % (by mass fraction                (Q20 = 0,89 g/ml).

- Ammonium oxalate, cold saturated solution.

- Citric acid monohydrate, 300 g/l solution.

- Ammonium chloride, 50 g/l solution.

- Bromocresol green, 0,4 g/l solution.

- Potassium permanganate, standard volumetric solution concentration (1/5 KMnO4) = 0,1 mol/l.

- Apparatus

- Electric muffle furnace, with air circulation, capable of being maintained at 550 ± 20 °C.

- Incineration dish, made of platinum, silica or porcelain.

- Sintered glass filter crucible, of porosity grade P 16 (pore size 10 to 16 µm).

d) 	Boiling water-bath.

- Beakers, of capacity 250 ml.

- Volumetric flask, capacity 250 ml.

g) 	Analytical balance.

- Sampling

Take the laboratory sample in accordance with ZWS 530.

- Procedure

  - Preparation of the Test Sample

Prepare the test sample in accordance with ZWS 530.

  - Test Portion

Weigh, to the nearest one milligram, approximately 5 g of the test sample, or more if necessary, into the incineration dish [4 (b)].

6.3	Determination

6.3.1	Ash the test portion in the electric muffle furnace [4 (a)] maintained at 550 ± 20 °C, until all organic matter has been destroyed (usually 4 h is sufficient).

If some organic matter remains (black particles), add a few drops of nitric acid [3 (b)], dry on a hotplate and ash again in the muffle furnace at 550 ± 20 °C for 30 min. Repeat this until all organic matter has been destroyed. Transfer the ash to a 250 ml beaker [4 (e)].

6.3.2	Add 60 ml of water, 40 ml of the hydrochloric acid [3 (a)], and a few drops of the nitric acid [3 (b)]. Bring to the boil and boil for 30 min. Cool and transfer the solution to a 250 ml one-mark volumetric flask [4 (f)]. Rinse, dilute to the mark with water, mix and filter to give the test solution.

6.3.3 	By means of a pipette, transfer an aliquot portion of the test solution (6.3.2), containing 10 to 40 mg of calcium, according to the expected calcium content, to a 250 ml beaker [4 (e)]. Add one millilitre of the citric acid solution [3 (f)] and 5 ml of the ammonium chloride solution [3 (g)]. Dilute to approximately 100 ml with water. Bring to the boil, add 10 drops of bromocresol green solution [3 (h)] and 30 ml of a warm solution of the ammonium oxalate [3 (e)]. If a precipitate forms, dissolve it by adding a few drops of the hydrochloric acid [3 (a)].

Neutralize very slowly with the ammonia solution [3 (d)], stirring continuously, until a pH of 4,4 to 4,6 is reached (i.e when the indicator changes colour). Place the beaker in a boiling water-bath [4 (d)] and 
leave for 30 min to allow the precipitate which has formed to settle. Remove the beaker from the water bath. Leave for one hour and filter through the filter crucible [4 (c)].

Wash the beaker and the crucible with water until the excess ammonium oxalate is completely removed as shown by the absence of chloride in the washing water.

Place the crucible in a 250 ml beaker [4 (e)] or wide-mouthed flask. Add 80 ml of the sulfuric acid [3 (c)] and heat to 70 to 80 °C to dissolve the precipitate.

6.3.4	Titrate the hot solution with the standard volumetric potassium permanganate solution [3 (i)], until a pink colour, persisting for one minute, is obtained.

7. 	Expression of Results

7.1	Method of Calculation and Formula

The calcium content, expressed in grams per kilogram of sample, is equal to

where

V 	is the volume, in millilitres, of the standard volumetric potassium permanganate solution used for the titration;

c 	is the exact concentration, in moles per litre, of the standard volumetric potassium permanganate solution;

m 	is the mass, in grams, of the test portion;

V′	is the volume, in millilitres, of the aliquot portion taken in 6.3.3.

Take as the result the arithmetic mean of the values obtained in the two determinations (see 6.4), provided that the requirement for repeatability (see 7.2) is satisfied.

Report the result to the nearest 1 g/kg.

7.2	Repeatability

The difference between the values obtained in the two determinations carries out simultaneously or in rapid succession by the same analyst shall not exceed:

1 g/kg (absolute) for calcium contents less than 50 g/kg;

2 % (relative) of the mean for calcium contents of 50 g/kg or more.

8. 	Notes on Procedure

8.1	For the determination of very low calcium contents use the atomic absorption method.

8.2	If the sample consists exclusively of mineral substances, dissolve it in hydrochloric acid without ashing.

For products such as aluminocalcium phosphates, which are difficult to dissolve in acids, mix the test portion in a platinum dish with five times its mass of a mixture comprising equal parts of potassium carbonate and sodium carbonate. Heat carefully until the mixture is completely molten. After cooling, dissolve in the hydrochloric acid.

8.3	If the magnesium content of the sample is likely to exceed the calcium content, or in cases of doubt, precipitate the calcium oxalate a second time.

9.	Test Report

The test report shall show the method used and the result obtained. It shall also mention any operating conditions not specified in this standard, or regarded as optional, as well as any circumstances that may have influenced the results.

The test report shall include all the details required for the complete identification of the sample.